Project Proposal

Spring 2025

Group Members:

Anees Ur Rehman (22PWCSE2168)

Faizan (22PWCSE2134)

Waqas Atta (22PWCSE2142)

Zeeshan Ahmad (22PWCSE2217)

Class Section: B

Subject: DSD LAB

“On my honor, as student of University of Engineering and Technology, I have neither given nor received unauthorized assistance on this academic work.”

Submitted to:

Engr. Faheem Jan

Sunday, May 18, 2025

Department of Computer Systems Engineering

University of Engineering and Technology, Peshawar

# Project Proposal

## 1. Project Title

Real-Time Temperature Monitoring System Using FPGA

## 2. Introduction

Accurate temperature monitoring is critical in industrial automation, environmental control, and smart systems. This project aims to develop a real-time temperature monitoring system leveraging the Spartan 6 FPGA, interfacing with an analog LM35 temperature sensor via an ADC module. The system will digitize, process, and display temperature data, while providing alerts for critical temperature conditions.

## 3. Objectives

- Interface LM35 temperature sensor with Spartan 6 FPGA through an ADC module.

- Digitally process sensor data to obtain precise temperature values.

- Display real-time temperature readings on an LCD or seven-segment display.

- Implement alert mechanisms (LED/buzzer) for threshold-exceeding temperatures.

## 4. Working Principle

The LM35 sensor outputs an analog voltage proportional to temperature. This voltage is converted to digital form by an ADC (MCP3008) and sent to the FPGA via SPI. The FPGA processes the digital data, converts it into temperature readings, displays the data, and triggers alerts if temperature exceeds preset limits.

## 5. Methodology

Step 1: Sensor Integration

- Capture analog temperature with LM35 sensor.

- Convert analog signal to digital using MCP3008 ADC.

- Interface ADC to FPGA via SPI communication.

Step 2: FPGA Data Processing

- Implement Verilog HDL logic to read ADC data and convert to Celsius temperature.

- Continuously monitor and process temperature data.

Step 3: Display Output

- Drive LCD or seven-segment display to show live temperature readings.

Step 4: Alert System

- Configure threshold-based logic to activate LED or buzzer alerts.

## 6. Work Plan / Procedure

- Power up hardware and initialize ADC-FPGA communication.

- Continuously sample temperature via LM35 and ADC.

- FPGA receives, processes, and scales data to temperature values.

- Display current temperature on output device.

- Monitor temperature thresholds; trigger alert devices if exceeded.

- Repeat process for continuous real-time monitoring.

## 7. Resources Required

| Hardware | Software |
| --- | --- |
| Spartan 6 FPGA Development Board | Xilinx ISE Design Suite |
| LM35 Temperature Sensor | Verilog HDL |
| MCP3008 ADC Module |  |
| LCD / Seven-Segment Display |  |
| LED and Buzzer |  |
| Power Supply, Breadboard, Wires |  |

## 8. Expected Outcomes

- Functional real-time temperature monitoring with accurate digital readout.

- Reliable alert system for critical temperature conditions.

- Hands-on experience in FPGA design, sensor interfacing, and ADC integration.

- Foundation for advanced features like data logging or wireless communication.

## 9. Conclusion

The proposed FPGA-based temperature monitoring system offers an efficient, scalable, and real-time solution for temperature tracking with visual display and alert capabilities. It combines sensor interfacing, ADC data acquisition, and FPGA processing to provide a practical and educational embedded system platform.